LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY test_add16b IS

END test_add16b;

ARCHITECTURE behavior OF test_add16b IS

COMPONENT add16b

PORT(

A16     : IN  std_logic_vector(15 downto 0);

B16     : IN  std_logic_vector(15 downto 0);

Cin_16  : IN  std_logic;

S16     : OUT std_logic_vector(15 downto 0);

Cout_16 : OUT std_logic

);

END COMPONENT;

-- Inputs

signal A16 : std_logic_vector(15 downto 0) := (others => '0');

signal B16 : std_logic_vector(15 downto 0) := (others => '0');

signal Cin_16 : std_logic := '0';

-- Outputs

signal S16 : std_logic_vector(15 downto 0);

signal Cout_16 : std_logic;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: add16b PORT MAP (

A16 => A16,

B16 => B16,

Cin_16 => Cin_16,

S16 => S16,

Cout_16 => Cout_16

);

-- Test cases

A16 <= X"0000" after 0 ns, X"000F" after 100 ns, X"0F0F" after 200 ns, X"FFFF" after 300 ns;

B16 <= X"0000" after 0 ns, X"0001" after 100 ns, X"F0F0" after 200 ns, X"0001" after 300 ns;

Cin_16 <= '0' after 0 ns, '1' after 400 ns;

END;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity add16b is

Port (

A16     : in  STD_LOGIC_VECTOR(15 downto 0);

B16     : in  STD_LOGIC_VECTOR(15 downto 0);

Cin_16  : in  STD_LOGIC;

S16     : out STD_LOGIC_VECTOR(15 downto 0);

Cout_16 : out STD_LOGIC

);

end add16b;

architecture Behavioral of add16b is

Component add4b

Port (

A4     : in  STD_LOGIC_VECTOR(3 downto 0);

B4     : in  STD_LOGIC_VECTOR(3 downto 0);

Cin_4  : in  STD_LOGIC;

S4     : out STD_LOGIC_VECTOR(3 downto 0);

Cout_4 : out STD_LOGIC

);

end Component;

signal C : STD_LOGIC_VECTOR(4 downto 0); -- Propagation des retenues

begin

C(0) <= Cin_16;

Res1 : add4b PORT MAP(A16(3 downto 0), B16(3 downto 0), C(0), S16(3 downto 0), C(1));

Res2 : add4b PORT MAP(A16(7 downto 4), B16(7 downto 4), C(1), S16(7 downto 4), C(2));

Res3 : add4b PORT MAP(A16(11 downto 8), B16(11 downto 8), C(2), S16(11 downto 8), C(3));

Res4 : add4b PORT MAP(A16(15 downto 12), B16(15 downto 12), C(3), S16(15 downto 12), C(4));

Cout_16 <= C(4); -- Retenue finale

end Behavioral;